SECTION 10 51 13

METAL LOCKERS

PART 1 – GENERAL

1.01	SECTION INCLUDES

A.	Heavy Duty Welded Lockers.

B.	Locker benches.

1.02	REFERENCES

A.	ADAAG - Americans with Disabilities Act, Accessibility Guidelines.

1.03	SUBMITTALS

A.	Submit under provisions of Section 01 33 00.

B.	Manufacturer's data sheets on each product to be used, including:

1.	Preparation instructions and recommendations.

2.	Storage and handling requirements and recommendations.

3.	Installation methods.

C.	Shop Drawings: Show the following:

1.	Dimensioned drawings including plans, elevations, and sections to show locker locations and interfaces with adjacent substrates.

2.	Details of assembly, erection, anchorage and clearance requirements.

D.	Selection Samples:  For each finish product specified, two complete sets of color chips representing manufacturer's full range of available colors and finishes.

1.04	DELIVERY, STORAGE, AND HANDLING

A.	Store products in manufacturer's unopened packaging until ready for installation.

B.	Protect locker finish and adjacent surfaces from damage.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A.	Acceptable Manufacturer: Penco Products, Inc., which is located at: 1820 Stonehenge Drive, Greenville, NC 27858; Tel: 800-562-1000; Fax: 800-248-1555; Email: general@pencoproducts.com; Web: www.pencoproducts.com

B.	Lockers shall be GREENGUARD Gold Certified by UL Environment through the GREENGUARD Certification Program.

2.02	MATERIALS

- Steel: Prime grade mild cold-rolled sheet steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A1008.

- Steel: Sheet steel components shall be fabricated using zinc-coated steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A879.

- Bolts and Nuts: Zinc plated truss fin head bolts and hex nuts.

- Provide only metal lockers fabricated in the United States by a single domestic manufacturer.

2.03	HEAVY DUTY LOCKERS

A.	Heavy Duty Lockers: All locker body components made of cold rolled steel specially 		formed for added strength and rigidity and to ensure tight joints at fastening points.

1.	Lockers with Doors:  Penco All-Welded lockers, with no legs (standard).

2.	Lockers with Doors:  Penco All-Welded lockers, with 4 inch (101 mm) legs. 			(optional)

3.	Lockers with Doors:  Penco All-Welded lockers, with 4 inch (101 mm) integral 			base. (optional)

B.	Locker Body: All-Welded Lockers.

1.	Sides, Bottoms, Tops, and Shelves:

a.	16 gauge steel.

b.        Perforated sides

2.	Backs: Solid 18 gauge steel.

a.	Groups to 48 inches (1.219 m) wide:  One-piece back.

b.	Groups over 48 inches (1.219 m) wide:  Two-piece back.

3.	Doors:

a.	14 gauge steel.

b.	Ventilation:  3/4 inch (19 mm) wide by 1-1/2 inch (38 mm) high 					diamond-shaped perforations.

c.	Solid doors.

4.	Sides: Vertical frames and sides.

a.	Intermediate vertical side frames:  Frame channel securely welded to 				locker side.

b.	Ventilation:  3/4 inch (19 mm) wide by 1-1/2 inch (38 mm) high 					diamond-shaped perforations.

c.	No Ventilation: Solid Sides.

5.	Tops: Notched and formed sheet; one continuous flat top for each group of 			lockers.

6.	Bottoms: Notched and formed sheet; one continuous bottom for each group 			of lockers, suitable for anchoring to wood or concrete bases.

7.	Shelves:  Flanged four sides with additional return flange on front edge to 			increase strength.

8.	Door frames, 16 gauge formed in a channel shape with continuous vertical 			door strikes.

C.	Locker Doors: One piece sheet steel.

1.	Multi-Point Latch Doors:  Full channel formation of adequate depth to fully 			conceal lock bar on lock side, channel formation on hinge side, right angle 			formations across top and bottom, with holes for attaching number plates.

2.	Single Point Latch Doors: 14 gauge door reinforced by a full height 3-1/2 inch 			(89 mm) wide, 18 gauge vertical pan welded to the top, bottom and hinge side 			flanges and rear of door skin on 12 inch (305 mm), 15 inch (381 mm) and 18 			inch (457 mm) wide doors. Provide a horizontal pan for doors wider than 18 			inches (457 mm).

3.	Single Point Latch Doors: 14 gauge door reinforced by a full 18 gauge inner 			pan welded to outer door skin on all four sides.

a.	Solid outer door, solid inner pan

b.	Diamond perforated outer, offset diamond perforated inner pan.

c.	3 inch or 6 inch louvered outer door, mini louvered inner pan.

d.	Mini louvered outer, mini louvered inner pan.

4.	Doors over 15 inches (380 mm) Wide and 30 inches (0.762 m) High: Provided 			with 3 inch (75 mm) wide 20 gauge full height reinforcing pan welded to inside 			face of door at 6 inch (150 mm) centers.

5.	Box Lockers (4 to 9 Tiers):  Channel formations on lock and hinge sides; right 			angle flanges on top and bottom with friction catch door pull.

6.	Provide holes for attaching number plates.

7.	Ventilation:

a.	Doors 20 inches (508 mm) or higher:  3/4 inch (19 mm) wide by 1-1/2 				inch (38 mm) high diamond-shaped perforations.

b.	All other doors:  7/16 inch (11 mm) wide by 15/16 inch (24 mm) high 				diamond-shaped perforations.

8.	Ventilation: Provide louvered doors in manufacturer's standard louver 				pattern.

9.	Ventilation: Provide louvered doors in manufacturer's standard mini-louver 			pattern, louvers 5/8 inch (15.8 mm) wide and 1/4 inch (6.3 mm) high.

10.	Ventilation: Provide fully louvered doors in manufacturer's standard full louver 			pattern.

11.	No Ventilation: Provide with solid doors.

D.	Hinges:

1.	Continuous Hinges: Continuous piano hinge for the full height of the door.

E.	Legs:  Furnish each group of lockers with four 4-inch (101 mm) high 14 gauge steel 		legs welded to locker bottom.

F.	Integral Metal Base:  4-inch (101 mm) high 16 gauge steel channel, welded to the 		locker bottom.

2.04	DOOR HANDLES AND LATCHING

A.	1, 2 and 3 Tier Lockers:

1.	Single-point latching:

a.	Recess handle in door.

b.	Integral Pocket and Pull:  22 gauge brushed stainless steel securely 				fastened to door with two lugs and a positive tamper-resistant 					decorative fastener.

1)	Pocket Depth:  Sufficient to prevent a combination padlock, built-					in combination lock, or key lock from protruding beyond door 					face.

2)	Pull:  Formed in pocket.

3)	Padlock Staple:  Protruding through pocket.

c.	Provide lock hole cover plate for use with padlocks.

d.	Locking Device:  11 gauge steel hasp welded to locker frame; include 				surface for engaging the bolt of a built-in combination or key lock and 				anti-pry lug and slot to deter prying open when locked.

e.	Firmly secure rubber silencers to locker frame.

2.	Multi-point latching with recessed handles:

a.	Recess finger-lift control handle in door.

b.	Pocket: 22 gauge brushed stainless steel securely fastened to door 				with two tabs and a positive tamper-resistant decorative fastener; of 				depth sufficient to prevent a combination padlock, built-in combination 				lock, or key lock from protruding beyond door face.

c.	Provide lock hole cover plate for use with padlocks.

d.	Attach 14 gauge formed steel lifting piece to latching channel with one 				concealed retaining lug and one rivet, assuring a positive two-point 				connection.

e.	Handle Finger Lift: Molded, sound-deadening, attached with rivet; 				padlock eye for use with 9/32 inch (7.1 mm) diameter padlock shackle.

f.	Latch Clip:  Glass-filled nylon engaging the door frame and holding the 				door shut.

1)	Doors 60 inches (1.524 m) and 72 inches (1.828 m) high:  Three 					points.

2)	Doors 20 inches (0.508 m) to 48 inches (1.22 m) high:  Two 					points.

g.	Locking Device: Positive, automatic type, whereby locker may be 				locked when open, then closed without unlocking.

h.	Firmly secure one rubber silencer in frame at each latch hook.

B.	1, 2 and 3 Tier Lockers:

1.	Single-point latching using double pan door:

a.	Recess handle in door.

b.	Integral Pocket and Pull:  22 gauge brushed stainless steel securely 				fastened to door with two lugs and a positive tamper-resistant 					decorative fastener.

1)	Pocket Depth: Sufficient to prevent a combination padlock, built-					in combination lock, or key lock from protruding beyond door 					face.

2)	Pull: Formed in pocket.

3)	Padlock Staple:  Protruding through pocket.

c.	Provide lock hole cover plate for use with padlocks.

d.	Locking Device:  11 gauge steel hasp welded to locker frame; include 				surface for engaging the bolt of a built-in combination or key lock and 				anti-pry lug and slot to deter prying open when locked.

e.	Firmly secure rubber silencers to locker frame.

2.	Cremone Latching Option: Handle shall be a heavy duty turn handle that 			engages the door frame on three sides. The top and bottom frames are 				engaged with 3/8 inch (9.5 mm) steel rods, and a 1/8 inch (3 mm) thick center 			latch engages at the side.

a.	Double door configurations shall consist of a left hinged door secured 				its full length by the right hinged door when latched.

b.	Handle Assembly shall be secured to door using a threaded lock nut to 				facilitate adjustment and removal for repair if necessary. Welded 					handle assemblies shall not be accepted.

3.       Single-Point Latching for wrap around/rotary locks:

a.	Recess Handle in door

b.	Integral Pocket and Pull:  22 gauge brushed stainless steel securely 		fastened to door with two lugs and a positive tamper-resistant 			decorative fastener.

1)	Pocket Depth: Sufficient to prevent a combination padlock, built-		in combination lock, or key lock from protruding beyond door 		face.

2)	Pull: Formed in pocket.

3) 	Pocket punched for built-in lock mounting only. No padlock slot.

c.	Locking Device: 11 gauge steel hasp welded to locker frame; include 		surface for secure wrap around vertical engagement by a built-in 			rotary-type lock to deter prying open when locked. Hasp must be 			adaptable to common horizontal throw locker lock and padlock use.

d.	Firmly secure rubber silencers to locker frame.

C.	Box Lockers (4 to 9 Tier):

1.	Punch doors for use with padlocks or built-in locks.

2.	Equip doors for use with padlocks with an 18 gauge combination door pull, 			staple, and lock hole cover plate with integral friction catch.

D.	Box Lockers (4 to 9 Tier):

1.	Finger-operated 11 gauge steel slam latch in each door with an 					electrogalvanized trigger and spring contained in a 14 gauge steel case 				welded to the door.

2.	Weld 13 gauge steel hasp to frame to engage spring latch.

3.	Securely install rubber bumpers in frame.

2.05	INTERIOR EQUIPMENT

A.	ADA-Compliant Lockers (Recessed Handles with Multi-Point Latch):

1.	Single Tier Lockers:  Hat shelf at maximum 48 inches (1.219 m) off the floor 			for unobstructed forward and side reach.

2.	Locker Compartment Bottom:  Minimum of 15 inches (230 mm) off the floor, 			or an extra shelf placed 15 inches (381 mm) off the floor for unobstructed 			forward and side reach.

3.	Handicapped symbol attached to door.

4.	Hooks and rods as specified for other lockers.

B.	All Welded Lockers:

1.	Single-Tier, 48-1/2 inches (1.232 m) or Higher: Hat shelf located 					approximately 9 inches (228 mm) below top of locker.

2.	Openings 20 inches (0.508 m) or Higher and 12 (305 mm) or 15 inches (380 			mm) Wide:  Two single-prong wall hooks and one double-prong ceiling hook.

3.	Over 15 inches (380 mm) Wide:  Four single-prong wall hooks and one 				double-prong ceiling hook.

2.06	ACCESSORIES

A.	Number Plates: Provide each locker with a polished aluminum number plate, 2-1/4 		inches (57 mm) wide by 1 inch (25 mm) high, with black numerals not less than 3/8 		inch (9.5 mm) high; attach to face of door with two aluminum rivets.

B.	Locks: Built-in flat key locks; control-key to same series.

C.	Locks: Built-in grooved key locks (pin tumbler); control-key to same series.

D.	Locks: Built-in three-number dialing combination locks capable of at least five 			different combinations changes; provide control key, combination change key, and 		combination control charts.

1.	Horizontal bolt

2.	Wrap around/rotary bolt

E.	Padlocks: Master-keyed three-number dialing combination type padlocks; provide 		master key.

F.	Coin-Operated Locks:

1.	Coin return/deposit type.

a.	Token.

b.	One quarter.

c.	Two quarters.

2.	Coin collect/pay type with cash box.

a.	Token.

b.	One quarter.

c.	Two quarters.

G.	Continuous Sloped Hoods: 18 gauge steel, slope rise equal to 1/3 of the locker 		depth (18.5 degrees), plus a 1 inch (25 mm) vertical rise at front.

1.	Supplied in 72 inch (1829 mm) lengths only.

2.	Slip joints without visible fasteners at splice locations.

3.	Provide necessary end closures.

4.	Finish to match lockers.

H.	Continuous Sloped Hoods: 16 gauge steel, slope rise equal to 1/3 of the locker 		depth (18.5 degrees), plus a 1 inch (25 mm) vertical rise at front.

1.	Supplied in 72 inch (1829 mm) lengths only.

2.	Slip joints without visible fasteners at splice locations.

3.	Provide necessary end closures.

4.	Finish to match lockers.

I.	Finished End Panels: Minimum 16 gauge steel formed to match locker depth and 		height, 1 inch (25 mm) edge dimension; finish to match lockers and install with 		concealed fasteners.

J.	Front Fillers: 20 gauge steel formed in an angle shape, with 20 gauge slip joint 		angles formed in an angle shape with double bend on one leg forming a pocket to 		provide adjustable mating with angle filler.

1.	Attachment by means of concealed fasteners.

2.	Finish to match lockers.

K.	Recess Trim: 18 gauge steel, 3 inch (75 mm) face dimension.

1.	Vertical and/or horizontal as required.

2.	Standard lengths as long as practical.

3.	Attach to lockers with concealed clips.

4.	Provide necessary finish caps and splices.

5.	Finish to match lockers.

L.	Benches:  Laminated selected hardwood, 1-1/4 inch (31 mm) full finished thickness, 		corners rounded and sanded, surfaces finished with two coats of clear lacquer.

1.	Depth: 9-1/2 inches (240 mm) deep.

2.	Depth: 12 inches (305 mm) deep.

3.	Depth: 24 inches (610 mm) deep.

4.	Lengths:  As shown.

M.	Heavy-Duty Bench Pedestals: Steel tubing with 11 gauge steel flanges welded to 		each end, 16-1/4 inches (412 mm) high, finish to match lockers.

N.	Stainless Steel Free-Standing Bench Pedestals: 2-inch (50 mm) diameter brushed 		16 gauge stainless steel formed into a trapezoid, 14 inch (355 mm) wide bottom with 		two 5/16 inch (7.9 mm) diameter holes, top flange with four 5/16 inch (7.9 mm) 		diameter holes for fastening to bench

2.07	FABRICATION

A.	Fabricate lockers square, rigid, without warp, with metal faces flat and free of 			distortion.

B.	Welded Lockers: Pre-assemble lockers by welding into one piece structures in 		groupings most practical for job requirements, welds free of burrs; maximum width 		of group to be 54 inches (1.371 m); no bolts, nuts, or rivets allowed in assembly of 		main locker groups.

C.	Finish: Enamel powder coat paint finish electrostatically applied and properly cured 		to manufacturer’s specifications for optimum performance. Finishes containing 		volatile organic compounds and subject to out-gassing are not acceptable. Locker 		exterior and interior shall be painted the same color.

1.	Powder Coat - Dry Thickness:  1 to 1.2 mils (0.025 to 0.03 mm).

2.	Powder Coat Plus - Dry Thickness:  2 to 2.2 mils (0.05 to 0.055 mm).

3.	Color:  As selected from manufacturer's standard colors.

4.	Special Finish

a.	Custom color

b.	Anti-Graffiti

c.	Anti-Microbial

d.	TGIC

e.	Ultra-Weatherable

PART 3 – EXECUTION

3.01	EXAMINATION

A.	Do not begin installation until substrates and bases have been properly prepared.

B.	If substrate and bases are the responsibility of another installer, notify Architect of 		unsatisfactory preparation before proceeding.

3.02	INSTALLATION

A.	Install metal lockers and accessories at locations shown in accordance with 			manufacturer's instructions.

B.	Install lockers plumb, level, and square.

C.	Anchor lockers to floor and wall at 48 inches (1.219 m) or less, as recommended by 		the manufacturer.

D.	Bolt adjoining locker units together to provide rigid installation.

E.	Install sloping tops and metal fillers using concealed fasteners.  Provide flush 			hairline joints against adjacent surfaces.

F.	Install front bases between legs without overlap or exposed fasteners.  Provide end 		bases on exposed ends.

G.	Install benches by fastening bench tops to pedestals and securely anchoring to the 		floor using appropriate anchors for the floor material.

3.03	ADJUSTING AND CLEANING

A.	Adjust doors and latches to operate without binding.  Verify that latches are 			operating satisfactorily.

B.       Adjust built-in locks to prevent binding of dial or key and ensure smooth operation 		prior to substantial completion.

C.	Touch-up with factory-supplied paint and repair or replace damaged products before 		substantial completion.

3.04	PROTECTION

A.	Protect installed products until completion of project.

END OF SECTION

Penco Products, Inc. reserves the right to vary specifications consistent with a policy of continuous product improvement.