SECTION 10 51 13

METAL LOCKERS

PART 1 – GENERAL

1.01	SECTION INCLUDES

A.	Heavy Duty Welded Gear Lockers.

B.	Locker benches.

1.02	REFERENCES

A.	ADAAG – Americans with Disabilities Act, Accessibility Guidelines.

1.03	SUBMITTALS

A.	Submit under provisions of Section 01 33 00.

B.	Shop Drawings: Show the following:

1.	Dimensioned drawings including plans, elevations, and sections to 				show locker locations and interfaces with adjacent substrates.

2.	Details of assembly, erection, anchorage and clearance requirements.

C.	Selection Samples:  For each finish product specified, two complete sets of 			color chips representing manufacturer's full range of available colors and 			finishes.

1.04	DELIVERY, STORAGE, AND HANDLING

A.	Store products in manufacturer's unopened packaging until ready for installation.

B.	Protect locker finish and adjacent surfaces from damage.

PART 2 – PRODUCTS

2.01	MANUFACTURERS

A.	Acceptable Manufacturer: Penco Products, Inc., which is located at: 1820 Stonehenge Drive, Greenville, NC 27858; Tel: 800-562-1000; Fax: 800-248-1555; Email: general@pencoproducts.com; Web: www.pencoproducts.com

B.	Lockers shall be GREENGUARD Gold Certified by UL Environment through the GREENGUARD Certification Program.

2.02	MATERIALS

- Steel: Prime grade mild cold-rolled sheet steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A1008.

- Steel: Sheet steel components shall be fabricated using zinc-coated steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A879.

- Bolts and Nuts: Zinc plated truss fin head bolts and hex nuts.

- Provide only metal lockers fabricated in the United States by a single domestic manufacturer.

2.03	HEAVY DUTY PERSONAL GEAR LOCKERS

A.	Heavy Duty Lockers: All locker body components made of cold rolled steel specially 		formed for added strength and rigidity and to ensure tight joints at fastening points.

B.	 Locker Body: All-Welded Gear Lockers.

1.	Sides, Bottoms, Tops, and Shelves:

a.	16 gauge steel.

b.        Solid sides punched for shelf locations on nominal 12 inch centers

2.	Backs:  Solid 18 gauge steel.

3.	Doors:

a.	14 gauge steel.

b.	Standard Ventilation: 6 inch (152.4 mm) wide by 3/4 inch (19 mm) high 				horizontal louvers arranged in two groups of six.

c.       Optional Ventilation: 3/4 inch (19 mm) wide by 1-1/2 inch (38 mm) high 				diamond-shaped perforations or 3/4 inch (19 mm) wide by 1/4 inch (6.4 				mm) high security mini louvers.

d.	Solid doors.

4.	Sides: Vertical frames and sides.

a.	Intermediate Vertical Side Frames:  Another frame channel securely 				welded to side frame.

b.	Ventilation:  3/4 inch (19 mm) wide by 1-1/2 inch (38 mm) high 					diamond-shaped perforations.

c.       Optional: Solid Sides

5.	Tops: Notched and formed sheet; one continuous flat top for each group of 			lockers.

6.	Channel Base: Notched and formed 14 gauge sheet forming a full locker 			bottom and 4 inch (101 mm) integral base; one continuous bottom for each 			locker, suitable for anchoring to wood or concrete bases.

7.	Shelves:  Flanged four sides with additional return flange on front edge to 			increase strength.

8.	Door frames, 16 gauge formed in a channel shape with continuous vertical 			door strikes.

a.	Vertical frame members shall be integral with locker side panel. 					Separate frame and side pieces fastened by welding or nut and bolt 				are not acceptable

C.	Locker Doors: One piece sheet steel.

1.	Provide holes for attaching number plates.

2.	Ventilation:  Provide louvered doors in manufacturer's standard louver 				pattern.

D.	Hinges:

1.	Continuous type:  16 gauge piano hinge measuring full height of door. 				Welded to door and attached to locker frame using steel rivets.

2.04	DOOR HANDLES AND LATCHING

A.	Single Tier Lockers:

1.	Cremone Latching: Handle shall be a heavy duty turn handle that 				engages the door frame on three sides. The top and bottom frames are 				engaged with 3/8 inch (9.5 mm) steel rods, and a 1/8 inch (3 mm) thick 				center latch engages at the side.

a.       Double door configurations shall consist of a left hinged door 					secured its full length by the right hinged door when latched.

b.       Handle Assembly shall be secured to door using a threaded lock 					nut to facilitate adjustment and removal for repair if necessary. 					Welded handle assemblies shall not be accepted.

2.	Single-point latching, 18 inch and 24 inch wide single door only:

a.	Recess handle in door.

b.	Integral Pocket and Pull:  22 gauge brushed stainless steel 					securely fastened to door with two lugs and a positive tamper-					resistant decorative fastener.

1)	Pocket Depth:  Sufficient to prevent a combination 							padlock, built-in combination lock, or key lock from 							protruding beyond door face.

2)	Pull:  Formed in pocket.

3)	Padlock Staple:  Protruding through pocket.

c.	Provide lock hole cover plate for use with padlocks.

d.	Locking Device:  11 gauge steel hasp welded to locker frame; 					include surface for engaging the bolt of a built-in combination or 					key lock and anti-pry lug and slot to deter prying open when 					locked.

e.       Door shall be reinforced using a 20 gauge horizontal box pan 					MIG-welded a maximum of 8" on center to the hinge and latch 					side channels, and spot welded a maximum of 8” on center to 					the door skin along the upper and lower pan edge for optimum 					strength, quiet operation and stiffness. Pan shall completely 					enclose and reinforce the rear of the recessed pocket.

f.	Firmly secure rubber silencers to locker frame.

3.	Multi-point latching with recessed handle, 18 inch and 24 inch wide single door only:

a.	Recess finger-lift control handle in door.

b.	Pocket:  22 gauge brushed stainless steel securely fastened to 					door with two tabs and a positive tamper-resistant decorative 					fastener; of depth sufficient to prevent a combination padlock, 					built-in combination lock, or key lock from protruding beyond 					door face.

c.	Provide lock hole cover plate for use with padlocks.

d.	Attach 14 gauge formed steel lifting piece to latching channel 					with one concealed retaining lug and one rivet, assuring a 						positive two-point connection.

e.	Handle Finger Lift:  Molded, sound-deadening, attached with 					rivet; padlock eye for use with 9/32 inch (7.1 mm) diameter 					padlock shackle.

f.	Latch Clip:  Glass-filled nylon engaging the door frame and 					holding the door shut.

1)	Doors 72 inches (1.828 m) High:  Three points.

g.	Locking Device:	Positive, automatic type, whereby locker 					may be locked when open, then closed without unlocking.

h.	Firmly secure one rubber silencer in frame at each latch hook.

2.05	INTERIOR EQUIPMENT

A.	Heavy Duty All-Welded Gear Lockers With Doors:

1.	Single-Tier, 72 inches (1.828 m):  Shelf located approximately 12 inches 			(304.8 mm) below top of locker.

2.	Openings 24 inches (609.6 mm) deep:  Two heavy duty 7/8 inch x 2-1/4 inch 			x 3 inch single prong wall hooks and one 1 inch diameter coat rod.

2.06	ACCESSORIES

A.	Number Plates:  Provide each locker with a polished aluminum number plate, 2-1/4 		inches (57 mm) wide by 1 inch (25 mm) high, with black numerals not less than 3/8 		inch (9.5 mm) high; attach to face of door with two aluminum rivets.

B.       Optional Name Card Holder: Plated steel. Holds 1.25” (31.75 mm) x 2.75” card 		(69.85 mm).

C.	Locks: Built-in flat key locks; master-key to same series.

D.	Locks: Built-in grooved key locks (pin tumbler); master-key to same series.

E.	Locks: Built-in three-number dialing combination locks capable of at least five 			different combination changes; provide master key, combination change key, and 		combination control charts.

F.	Padlocks: Master-keyed three-number dialing combination type padlocks; provide 		master key.

G.       Interior Equipment

1.	Optional Center Partition: 16 gauge with beaded front. Attached securely to 			shelf and locker bottom or additional lower shelf. (Not for use with footlocker 			or seat) Mounting holes for half shelves provided on nominal 12 inch centers.

2.	Optional Acrylic mirror: Self adhesive, 6 inch (152.4 mm) x 8 inch (203.2 mm).

3. 	Optional extra full width shelves. 16 gauge, flanged on four sides and 				attached securely to solid locker side. Shelves over 24” wide are also 				attached to the locker back and punched for attachment of center divider.

4. 	Optional Half Shelves: 16 gauge, flanged on four sides and attached securely 			to center partition and locker side.

5. 	Optional Security box: 14 gauge lockable door with a 16 gauge side panel. 			The door is attached to a welded frame with a continuous hinge. The hinge is 			mounted to door with aluminum rivets. The door is locked through a single 			point latch with a padlock or built in lock. A door pull shall be provided for use 			with padlocks. Security box door frame to be not less than 16 gauge formed 			to a channel shape. Vertical members to have an additional flange to provide 			a continuous door strike. Intermembering parts to be mortised and tenoned 			electrically welded together in a rigid assembly capable of resisting strains.

a.	12 inch wide

b.	24 inch wide with full width interior shelf

6. 	Optional cell phone/key tray: 8 inch (203.2 mm) x 2 inch (50.8 mm) x 2 inch 			(50.8 mm) mounted to inside of 24” wide security box door.

7. 	Optional metal seat only: 14 gauge, located 18 inches from floor.

8. 	Optional hardwood seat only:  1-1/4” thick full depth located 18” from floor.

9. 	Optional footlocker: Recessed from locker frame to clear full length locker 			door. Front foot locker panel includes single point latch with padlock strike 			plate and mini louvers. Lid of foot locker has a continuous hinge and also 			serves as a seat. Opening and closing of the lid is quieted by rubber bumpers 			mounted to the contact points. The seat lid is strengthened with two 				reinforcement channels welded to bottom. Two side 	seat supports are bolted 			to side panels and inserted in a support tab on the front locker panel for 				added strength.

10.	Optional sheet steel document sleeve. 16 gauge measuring 12 inches wide 			(horizontal storage) or 9-1/2 inches wide (vertical storage) and attached using 			supplied hardware.

11.	Optional Garment separator. Formed from16 gauge sheet steel and 				perforated. Mounted underneath the full width shelf.

12.	Optional Case and Drawer. Drawer body shall be constructed of 16 gauge 			sheet steel, mounted on full extension, ball bearing glides rated at 250 lb. 			capacity for smooth operation. Drawer shall feature an integral pull spanning 			the full width of the 	drawer body.

13.	Optional Parachute/body armor rack. Formed from ¾ inch diameter, heavy 			wall steel tube. Reinforced by 14 gauge gussets and supports and mounted 			securely to locker back. Replaces standard rear mounted coat hooks. 180 			pound capacity.

H.	Continuous sloped hoods: 16 gauge steel, slope rise equal to 1/3 of the locker 		depth (18.5 degrees), plus a 1 inch (25 mm) vertical rise at front.

1.	Supplied in 72 inch (1829 mm) lengths only.

2.	Slip joints without visible fasteners at splice locations.

3.	Provide necessary end closures.

4.	Finish to match lockers.

I.	Finished End Panels: Minimum 16 gauge steel formed to match locker depth and 		height, 1 inch (25 mm) edge dimension; finish to match lockers. Install with 			concealed fasteners.

J.	Front Fillers: 20 gauge steel formed in an angle shape, with 20 gauge slip joint 		angles formed in an angle shape with double bend on one leg forming a pocket to 		provide adjustable mating with angle filler.

1.	Attachment by means of concealed fasteners.

2.	Finish to match lockers.

K.	Recess Trim: 18 gauge steel, 3 inch (75 mm) face dimension.

1.	Vertical and/or horizontal as required.

2.	Standard lengths as long as practical.

3.	Attach to lockers with concealed clips.

4.	Provide necessary finish caps and splices.

5.	Finish to match lockers.

L.	Benches: Laminated selected hardwood, 1-1/4 inch (31 mm) full finished thickness, 		corners rounded and sanded, surfaces finished with two coats of clear lacquer.

1.	Depth: 9-1/2 inches (240 mm) deep.

2.	Depth: 12 inches (305 mm) deep.

3.	Depth: 24 inches (610 mm) deep.

4.	Lengths:  As shown.

M.	Heavy-Duty Bench Pedestals: Steel tubing with 10 gauge steel flanges welded to 		each end, 16-1/4 inches (412 mm) high, finish to match lockers.

N.	Stainless Steel Free-Standing Bench Pedestals: 2-inch (50 mm) diameter brushed 		16 gauge stainless steel formed into a trapezoid, 14 inch (355 mm) wide bottom with 		two 5/16 inch (7.9 mm) diameter holes, top flange with four 5/16 inch (7.9 mm) 		diameter holes for fastening to bench

2.07	FABRICATION

A.	Fabricate lockers square, rigid, without warp, with metal faces flat and free of 			distortion.

B.	Finish:  Enamel powder coat paint finish electrostatically applied and properly cured 		to manufacturer’s specifications for optimum performance. Finishes containing 		volatile organic compounds and subject to out-gassing are not acceptable. Locker 		exterior and interior shall be painted the same color.

1.	Powder Coat - Dry Thickness:  1 to 1.2 mils (0.025 to 0.03 mm).

2.	Powder Coat Plus - Dry Thickness:  2 to 2.2 mils (0.05 to 0.055 mm).

3.	Color:  As selected from manufacturer's standard colors.

4.	Special Finish

a.	Custom color

b.	Anti-Graffiti

c.	Anti-Microbial

d.	TGIC

e.	Ultra-Weatherable

PART 3 – EXECUTION

3.01	EXAMINATION

A.	Do not begin installation until substrates and bases have been properly prepared.

B.	If substrate and bases are the responsibility of another installer, notify Architect of 		unsatisfactory preparation before proceeding.

3.02	INSTALLATION

A.	Install metal lockers and accessories at locations shown in accordance with 			manufacturer's instructions.

B.	Install lockers plumb, level, and square.

C.	Anchor lockers to floor and/or wall at 36 inches (0.914 m) or less, as recommended 		by the manufacturer.

D.	Bolt adjoining locker units together to provide rigid installation.

E.	Install sloping tops and metal fillers using concealed fasteners.  Provide flush 			hairline joints against adjacent surfaces.

F.	Install benches by fastening bench tops to pedestals and securely anchoring to the 		floor using appropriate anchors for the floor material.

3.03	ADJUSTING AND CLEANING

A.	Adjust doors and latches to operate without binding.  Verify that latches are 			operating satisfactorily.

B.	Adjust built-in locks to prevent binding of dial or key and ensure smooth operation 		prior to substantial completion.

C.	Touch-up with factory-supplied paint and repair or replace damaged products before 		substantial completion.

3.04	PROTECTION

A.	Protect installed products until completion of project.

END OF SECTION

Penco Products, Inc. reserves the right to vary specifications consistent with a policy of continuous product improvement.