SECTION 10 51 13

METAL LOCKERS

PART 1 – GENERAL

1.01 	SECTION INCLUDES

A. 	Standard Duty Knocked Down Lockers.

B. 	Locker benches.

1.02	REFERENCES

A.	ADAAG - Americans with Disabilities Act, Accessibility Guidelines.

1.03	SUBMITTALS

A.	Submit under provisions of Section 01 33 00.

B.	Manufacturer's data sheets on each product to be used, including:

1.	Preparation instructions and recommendations.

2.	Storage and handling requirements and recommendations.

3.	Installation methods.

C.	Shop Drawings: Show the following:

1.	Dimensioned drawings including plans, elevations, and sections to show locker locations and interfaces with adjacent substrates.

2.	Details of assembly, erection, anchorage and clearance requirements.

D.	Selection Samples:  For each finish product specified, two complete sets of color chips representing manufacturer's full range of available colors and finishes.

1.04	DELIVERY, STORAGE, AND HANDLING

A.	Store products in manufacturer's unopened packaging until ready for installation.

B.	Protect locker finish and adjacent surfaces from damage.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A.	Acceptable Manufacturer: Penco Products, Inc., which is located at: 1820 Stonehenge Drive, Greenville, NC 27858; Tel: 800-562-1000; Fax: 800-248-1555; Email: general@pencoproducts.com; Web: www.pencoproducts.com

B.	Lockers shall be GREENGUARD Gold Certified by UL Environment through the GREENGUARD Certification Program.

2.02 MATERIALS

- Steel: Prime grade mild cold-rolled sheet steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A1008.

- Steel: Sheet steel components shall be fabricated using zinc-coated steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A879.

- Bolts and Nuts: Zinc plated truss fin head bolts and hex nuts.

- Provide only metal lockers fabricated in the United States by a single domestic manufacturer.

2.03	STANDARD DUTY LOCKERS

A.	Standard Duty Lockers:

1.	Acceptable Product: Penco Guardian, knock-down lockers.

2.	Tops, Bottoms, Backs, Sides, and Shelves: 24 gauge sheet steel.

3.	Doors over 12 inches (305 mm) wide or 20 inches (760 mm) high:  16 gauge 			sheet steel.

4.	Doors 12 inches or less (305 mm) wide: 18 gauge sheet steel.

5.	Legs: 6 inches (150 mm) high (standard).

6.	No legs (optional).

B. 	Locker Body: Steel specially formed for added strength and rigidity and to ensure 		tight joints at fastening points.

1.	Tops and bottoms with three sides formed 90 degrees, the front offset formed 			to be flush with horizontal frame member.

2.	Shelves with four sides formed to 90 degrees, front edge having a second 			bend.

3.	Hole spacing in locker body construction:  Not exceeding 9 inches (225 mm).

4.	Form door frame members to a channel shape, not less than 16 gauge steel.

5.	Provide vertical door frame members with additional 3/8 inch (9.5 mm) flange 			as a continuous door strike.

6.	Mortise and tenon intermembering parts; electrically weld together in a rigid 			assembly capable of resisting strains.

7.	Securely weld cross frame members of channel shapes to vertical framing 			members to ensure rigidity, including intermediate cross frame on double and 			triple tier lockers.

8.	Optional factory assembly of locker bodies using rivets.

9.	Center partitions:  24 gauge steel vertical partitions, full depth between 				bottom and shelf.

C.	Locker Doors: One piece sheet steel.

- Multi-Point Latch Doors: Full channel formation of adequate depth to fully 		conceal lock bar on lock side, channel formation on hinge side, right angle 		formations across top and bottom, with holes for attaching number plates.

- Single Point Latch Doors: 14 gauge door reinforced by a full height 3-1/2 inch 		(89 mm) wide, 18 gauge vertical pan welded to the top, bottom and hinge side 		flanges and rear of door skin on 12 inch (305 mm), 15 inch (381 mm) and 18 		inch (457 mm) wide doors. Provide a horizontal pan for doors wider than 18 		inches (457 mm).

- Single Point Latch Doors: 14 gauge door reinforced by a full 18 gauge inner 		pan welded to outer door skin on all four sides.

  - Solid outer door, solid inner pan

  - Diamond perforated outer, offset diamond perforated inner pan.

  - 3 inch or 6 inch louvered outer door, mini louvered inner pan.

  - Mini louvered outer, mini louvered inner pan.

- Provide holes for attaching number plates.

- Doors over 15 inches (380 mm) wide by 60 inches (1.524 m) or 72 inches 		(1.828 m) high:  3 inch (75 mm) wide 20 gauge full height reinforcing pan 		welded to inside face of door at 6 inch (150 mm) centers.

- Ventilation:  Flush door front with no exposed louvers and air flow slots  			located in top and bottom flanges of door.

D.	Hinges:

1.	Two inch high, 0.074 inch (1.88 mm) thick sheet steel, double spun, full loop, 			tight pin, projection welded to door frame and securely fastened to the door 			with two steel rivets.

a.	Doors over 48 inches (1.066 m) high:  Three 2 inch (51 mm) high five-			knuckle hinges.

b.	Doors over 24 inches (1.066 m) wide:  Four 2 inch (51 mm) high five-			knuckle hinges.

c.	All other doors:  Two 2 inch (51 mm) high five-knuckle hinges.

2.	16 gauge, 0.0625 inch (1.58 mm) thick sheet steel, continuous type hinge 		riveted to the frame and welded to the door.

2.04	DOOR HANDLES AND LATCHING

A.	Two Person and Duplex Lockers, 1, 2 and 3 Tier: Multi-point latching:

1.	Classic III Multi-point latching with recessed handles:

a.	Recess finger-lift control handle in door.

b.       Pocket: brushed stainless steel securely fastened to door with two tabs    and a positive tamper-resistant decorative fastener; of depth sufficient to prevent a combination padlock, built-in combination 	lock, or key lock from protruding beyond door face.

c.	Provide lock hole cover plate for use with padlocks.

d.	Attach 14 gauge formed steel lifting piece to latching channel with one 				concealed retaining lug and one rivet, assuring a positive two-point 				connection.

e.	Handle finger lift:  Molded, sound-deadening, attached with rivet; 					padlock eye for use with 9/32 inch (7.1 mm) diameter padlock shackle.

f.	Latch Clip:  Glass-filled nylon engaging the door frame and holding the 				door shut.

1)	Doors 60 inches (1.524 m) and 72 inches (1.828 m) high:  Three 					points.

2)	Doors 20 inches (0.508 m) to 48 inches (1.22 m) high:  Two 					points.

g.	Locking Device: Positive, automatic type, whereby locker may be 				locked when open, 	then closed without unlocking.

h.	Firmly secure rubber silencers in frame as required.

B.	1, 2 and 3 Tier: Single-point latching.

      - Single-point latching for horizontal bolt or padlock:

        - Recess handle in door.

        - Integral Pocket and Pull:  brushed stainless steel securely fastened to door with two lugs and a positive tamper-resistant decorative fastener.

          - Pocket Depth:  Sufficient to prevent a combination padlock, built-in combination lock, or key lock from protruding beyond door face.

          - Pull:  Formed in pocket.

          - Padlock Staple:  Protruding through pocket.

        - Provide lock hole cover plate for use with padlocks.

        - Locking Device:  11 gauge steel hasp welded to locker frame; include surface for engaging the bolt of a built-in combination or key lock and anti-pry lug and slot to deter prying open when locked.

        - Firmly secure rubber silencers to locker frame.

2.	Single-Point latching for wrap around/rotary latch locks:

a.	Recess Handle in door

b.	Integral Pocket and Pull:  brushed stainless steel securely 		fastened to door with two lugs and a positive tamper-resistant 			decorative fastener.

1)	Pocket Depth:  Sufficient to prevent a combination padlock, built-		in combination lock, or key lock from protruding beyond door 		face.

2)	Pull:	Formed in pocket.

3) 	Pocket punched for built-in lock mounting only. No padlock slot.

c.	Locking Device:  11 gauge steel hasp welded to locker frame; include 		surface for secure wrap around vertical engagement by a built-in 			rotary-type lock to deter prying open when locked. Hasp must be 			adaptable to common horizontal throw locker lock and padlock use.

d.	Firmly secure rubber silencers to locker frame.

2.05	INTERIOR EQUIPMENT

A.	ADA-Compliant Lockers (Recessed Handles with Multi-Point Latch):

1.	Single Tier Lockers:  Hat shelf 48 inches (1.219 m) maximum off the floor.

2.	Locker Compartment Bottom:  Minimum of 15 inches (230 mm) off the floor, 			or an extra shelf placed 15 inches (381 mm) off the floor for unobstructed 			forward and side reach.

3.	Handicapped symbol attached to door.

4.	Hooks and rods as specified for other lockers.

2.06	ACCESSORIES

A.	Number Plates: Provide each locker with a polished aluminum number plate, 2-1/4 		inches (57 mm) wide by 1 inch (25 mm) high, with black numerals not less than 3/8 		inch (9.5 mm) high; attach to face of door with two aluminum rivets.

B.	Closed Bases: 18 gauge closed metal front and end bases, finished to match

lockers.

C.	Locks: Built-in flat key locks; control-key to same series.

D. 	Locks: Built-in grooved key locks (pin tumbler); control-key to same series.

E.	Locks: Built-in three-number dialing combination locks capable of at least five 			different combinations changes; provide control key, combination change key, and 		combination control charts.

1.	Horizontal bolt

2.	Wrap around/rotary bolt

F. 	Padlocks: Control-keyed, three-number dialing combination type padlocks; provide 		control key. Mechanism must be resistant to “shimming”.

G.	Coin-Operated Locks:

1.	Coin return/deposit type.

a.	One quarter.

b.	Two quarters.

2.	Coin collect/pay type with cash box.

a.	One quarter.

b.	Two quarters.

.

H.	Continuous Sloped Hoods: 18 gauge steel, slope rise equal to 1/3 of the locker 		depth (18.5 degrees), plus a 1 inch (25 mm) vertical rise at front.

1.	Supplied in 72 inch (1829 mm) lengths only.

2.	Slip joints without visible fasteners at splice locations.

3.	Provide necessary end closures.

4.	Finish to match lockers.

I.        Continuous Sloped Hoods: 16 gauge steel, slope rise equal to 1/3 of the locker 		depth (18.5 degrees), plus a 1 inch (25 mm) vertical rise at front.

1.	Supplied in 72 inch (1829 mm) lengths only.

2.	Slip joints without visible fasteners at splice locations.

3.	Provide necessary end closures.

4.	Finish to match lockers.

J.	Unit Slope Tops for Standard Duty Lockers: 24 gauge steel, slope rise equal to 1/3 		of the locker depth, finish to match lockers.

K.	Finished End Panels: Minimum 16 gauge steel formed to match locker depth and 		height, 1 inch (25 mm) edge dimension; finish to match lockers; install with 			concealed fasteners.

L.	Front Fillers: 20 gauge steel formed in an angle shape, with 20 gauge slip joint 		angles formed in an angle shape with double bend on one leg forming a pocket to 		provide adjustable mating with angle filler.

1.	Attachment by means of concealed fasteners.

2.	Finish to match lockers.

M.	Zee Bases for Knock-Down Lockers without legs: 14 gauge, steel flanged outward 		at top for support of lockers, flanged inward at bottom for anchoring to floor.

1.	Height:  4 inches (101 mm).

2.	Height:  6 inches (150 mm).

N.	Recess Trim: 18 gauge steel, 3 inch (75 mm) face dimension.

1.	Vertical and/or horizontal as required.

2.	Standard lengths as long as practical.

3.	Attach to lockers with concealed clips.

4.	Provide necessary finish caps and splices.

5.	Finish to match lockers.

O.	Benches: Laminated selected hardwood, 1-1/4 inch (31 mm) full finished thickness, 		corners rounded and sanded, surfaces finished with two coats of clear lacquer.

1.	Depth: 9-1/2 inches (240 mm) deep.

2.	Depth: 12 inches (305 mm) deep.

3.	Depth: 24 inches (610 mm) deep.

4.	Lengths:  As shown.

P.	Heavy-Duty Bench Pedestals: Steel tubing with 11 gauge steel flanges welded to 		each end, 16-1/4 inches (412 mm) high, finish to match lockers.

Q.	Stainless Steel Free-Standing Bench Pedestals: 2-inch (50 mm) diameter brushed 		16 gauge stainless steel formed into a trapezoid, 14 inch (355 mm) wide bottom with 		two 5/16 inch (7.9 mm) diameter holes, top flange with four 5/16 inch (7.9 mm) 		diameter holes for fastening to bench

2.07	FABRICATION

A.	Fabricate lockers square, rigid, without warp, with metal faces flat and free of 			distortion.

B.	Knock-Down Lockers: Fabricate lockers on the unit principle, each locker with 			individual door and 	frame, individual top, bottom, back, and shelves, with common 		intermediate divisions separating compartments. Verify dimensions and 				arrangement before fabrication.

C.	Finish: Enamel powder coat paint finish electrostatically applied and properly cured 		to manufacturer’s specifications for optimum performance. Finishes containing 		volatile organic compounds and subject to out-gassing are not acceptable.

      - Powder Coat - Dry Thickness:  1 to 1.2 mils (0.025 to 0.03 mm).

      - Powder Coat Plus - Dry Thickness:  2 to 2.2 mils (0.05 to 0.055 mm).

      - Color:  As selected from manufacturer's standard colors.

      - Special Finish

        - Custom color

        - Anti-Graffiti

        - Anti-Microbial

        - TGIC

        - Ultra-Weatherable

PART 3 – EXECUTION

3.01	EXAMINATION

A.	Do not begin installation until substrates and bases have been properly prepared.

B.	If substrate and bases are the responsibility of another installer, notify Architect of 		unsatisfactory preparation before proceeding.

3.02	INSTALLATION

A.	Install metal lockers and accessories at locations shown in accordance with 			manufacturer's instructions.

B.	Install lockers plumb, level, and square.

C.	Anchor lockers to floor and wall at 48 inches (1.219 m) or less, as recommended by 		the manufacturer.

D.	Bolt adjoining locker units together to provide rigid installation.

E.	Install sloping tops and metal fillers using concealed fasteners.  Provide flush 			hairline joints against adjacent surfaces.

F.	Install front bases between legs without overlap or exposed fasteners.  Provide end 		bases on exposed ends.

G.	Install benches by fastening bench tops to pedestals and securely anchoring to the 		floor using appropriate anchors for the floor material.

3.03	ADJUSTING AND CLEANING

A.	Adjust doors and latches to operate without binding.  Verify that latches are 			operating satisfactorily.

B.       Adjust built-in locks to prevent binding of dial or key and ensure smooth operation 		prior to substantial completion.

C.	Touch-up with factory-supplied paint and repair or replace damaged products before 		substantial completion.

3.04	PROTECTION

A.	Protect installed products until completion of project.

END OF SECTION

Penco Products, Inc. reserves the right to vary specifications consistent with a policy of continuous product improvement.