SECTION 10 51 13

METAL LOCKERS

PART 1 – GENERAL

1.01	SECTION INCLUDES

A.	Heavy Duty Welded Fully Framed Turnout Open Front Lockers.

1.02	REFERENCES

A.	ADAAG – Americans with Disabilities Act, Accessibility Guidelines.

1.03	SUBMITTALS

A.	Submit under provisions of Section 01 33 00.

B.	Shop Drawings: Show the following:

1.	Dimensioned drawings including plans, elevations, and sections to show 			locker locations and interfaces with adjacent substrates.

2.	Details of assembly, erection, anchorage and clearance requirements.

C.	Selection Samples: For each finish product specified, two complete sets of color 		chips representing manufacturer's full range of available colors and finishes.

1.04	DELIVERY, STORAGE, AND HANDLING

A.	Store products in manufacturer's unopened packaging until ready for installation.

B.	Protect locker finish and adjacent surfaces from damage.

PART 2 – PRODUCTS

2.01	MANUFACTURERS

A.	Acceptable Manufacturer: Penco Products, Inc., which is located at: 1820 Stonehenge Drive, Greenville, NC 27858; Tel: 800-562-1000; Fax: 800-248-1555; Email: general@pencoproducts.com; Web: www.pencoproducts.com

B.	Lockers shall be GREENGUARD Gold Certified by UL Environment through the GREENGUARD Certification Program.

2.02	MATERIALS

- Steel: Prime grade mild cold-rolled sheet steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A1008.

- Steel: Sheet steel components shall be fabricated using zinc-coated steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A653.

- Flattened Expanded Metal Divisions that meets or exceeds ASTM A1008/A1008M, captured on all sides by the tubular shaped frame assembly

- Self-Tapping Screws and Rivets.

- Provide only metal lockers that conform to Made in US regulations.

2.03	HEAVY DUTY OPEN FRONT PERSONAL EQUIPMENT LOCKERS

A.	Heavy Duty Lockers: All locker body components made of cold rolled steel specially 		formed for added strength and rigidity and to ensure tight joints at fastening points.

B.	Locker Body Construction:

1.	Bottoms, Tops, and Shelves: 16-gauge steel.

2. 	Sides:

a.	16-gauge solid sides

b.	Flattened expanded metal

c. 	16-gauge diamond perforation on 72” high only

3.	Backs:  Solid 18-gauge steel. One-piece

4.	Tops and bottoms with four sides formed at 90 degrees welded to the fully- framed channel with cast corner pieces.

5.	Channel Base: 4-inch or 6-inch high. 14-gauge sheet forming a “C” channel attached to locker bottom.

6.	Shelves with four sides formed to 90 degrees, front and back edge having a second bend.

7.	Locker Body Construction: Welded shell. Tubular frame construction with cast steel interlocks provide a true fully framed locker body construction

8.	Hole spacing in locker body construction: Not exceeding 3-inches.

2.04	INTERIOR EQUIPMENT

A.	Heavy Duty Turnout Lockers Without Doors:

1.	Optional Full Width Shelf - located approximately 13-inches (330 mm) below top of locker.

2.	Optional Coat Rod – using two heavy duty single prong hooks, mounted to shelf and two single prong hooks with a diameter of 0.187 in rear, mounted to shelf.

2.05	ACCESSORIES

A.	Number Plates: Provide each locker with a polished aluminum number plate, 2-1/4 		inches (57 mm) wide by 1 inch (25 mm) high, with black numerals not less than 3/8 		inch (9.5 mm) high; attach to face of shelf with two aluminum rivets.

B.	Continuous sloped hoods: 16-gauge steel, slope rise equal to 1/3 of the locker 			depth (18.5 degrees), plus a 1-inch (25 mm) vertical rise at front.

1.	Supplied in 72-inch (1829 mm) lengths only.

2.	Slip joints without visible fasteners at splice locations.

3.	Provide necessary end closures.

4.	Finish to match lockers.

C.	Finished End Panels: Minimum 16-gauge steel formed to match locker depth and 		height, 1-inch (25 mm) edge dimension; finish to match lockers. Install with 			concealed fasteners.

D.	Front Fillers: 20-gauge steel formed in an angle shape, with 20-gauge slip joint 		angles formed in an angle shape with double bend on one leg forming a pocket to 		provide adjustable mating with angle filler.

1.	Attachment by means of concealed fasteners.

2.	Finish to match lockers.

E.	Recess Trim: 18-gauge steel, 3-inch (75 mm) face dimension.

1.	Vertical and/or horizontal as required.

2.	Standard lengths as long as practical.

3.	Attach to lockers with concealed clips.

4.	Provide necessary finish caps and splices.

5.	Finish to match lockers.

F.	Benches: Laminated selected hardwood, 1-1/4 inch (31 mm) full finished thickness, 		corners rounded and sanded, surfaces finished with two coats of clear lacquer.

1.	Depth: 9-1/2 inches (240 mm) deep.

2.	Depth: 12-inches (305 mm) deep.

3.	Depth: 24-inches (610 mm) deep.

4.	Lengths:  As shown.

G.	Heavy-Duty Bench Pedestals: Steel tubing with 11-gauge steel flanges welded to 		each end, 16-1/4 inches (412 mm) high, finish to match lockers.

H.	Stainless Steel Free-Standing Bench Pedestals: 2-inch (50 mm) diameter brushed 	16-gauge stainless steel formed into a trapezoid, 14-inch (355 mm) wide bottom

with two 5/16 inch (7.9 mm) diameter holes, top flange with four 5/16 inch (7.9 mm) 	diameter holes for fastening to bench

I.      Interior Equipment

1.	Optional Acrylic mirror: Self-Adhesive, 6-inch (152 mm) x 8-inch (203 mm).

2. 	Optional Full Width Shelves. 16-gauge, flanged on four sides and 		attached securely to mounting side rails.

3. 	Optional Center Partition: 16-gauge with flanged front. Attaches securely to shelf and locker bottom or additional lower shelf. (Not for use with footlocker or seat) Mounting holes for half shelves provided on nominal 6 inch centers.

4.	Optional Half Shelf: 16-gauge, used with full height center partition only. Flanged on three sides.

5.	Optional “L” Shaped Half Shelves: 16-gauge, flanged on three sides and attached securely on one side with a side rail, and flanged partition mounted to shelf or half shelf above.

6. 	Optional Security box: 16-gauge lockable door with a 16-gauge side panel. 	The door is attached to the welded frame of the security using a two-point

hinge. The door is locked through a single point latch with a padlock or built in

lock. A door pull shall be provided for use with padlocks. Security box door

frame to be not less than 16-gauge formed to a channel shape. Vertical

members to have an additional flange to provide a continuous door strike.

a.	9-inch wide

b.	12-inch wide

c.	15-inch wide

d. 	18-inch wide

e.	21-inch wide

c.	24-inch wide

7. 	Optional cell phone/key tray: 8-inch (203 mm) x 2-inch (51 mm) x 2-inch 			(51 mm) mounted to flanged partition side of the half shelf.

8. 	Optional metal seat only: 14-gauge, mounted using mounting rails to vertical corner posts allowing adjustability.

9. 	Optional hardwood seat only:  1-1/4” thick full depth mounted using mounting rails to vertical corner posts allowing adjustability.

10. 	Optional footlocker: Recessed from locker frame to clear full length locker 			door. Front foot locker panel includes single point latch with padlock strike 			plate and vertical slots. 14-gauge lid of footlocker has a continuous hinge and

Also serves as a seat. Opening and closing of the lid is quieted by rubber

bumpers mounted to the contact points. The seat lid is strengthened with	reinforcement channels welded to bottom. Lid/seat is supported by two

mounting side rails attached to the vertical posts.

11.	Optional sheet steel document sleeve. 16-gauge measuring 12-inches wide 		(horizontal storage) or 9-1/2 inches wide (vertical storage) mounted to flanged

partition side of the half shelf.

12.	Optional Drawers. 12-inch-high or 6-inch high. Drawer body shall be constructed of 16-gauge sheet steel, mounted using an integral “C” channel and mounted to the side rails that are mounted to the corner posts.

13.	Optional Parachute/body armor rack. Formed from ¾ inch diameter, heavy 		wall steel tube. Reinforced by 14-gauge gussets. Mounted to rear of locker

using two mounting rails between corner posts. 180 	pound capacity.

14.	Optional Inner Compartment Door, attached to “L” shaped half shelf

15.	Optional Coat Rods using coat rod hooks mounted to shelf

2.06	FABRICATION

- Fabricate lockers with fully framed construction, 8 male/female tubular insertion points to provide the fully framed locker body, square, rigid, without warp, with metal faces flat and free of distortion.

B.	Fabricate lockers on the unit principle, each locker with individual door and frame, individual top, bottom, back, and shelves. Verify dimensions and arrangement before fabrication.

C.	Finish:  Enamel powder coat paint finish electrostatically applied and properly cured 		to manufacturer’s specifications for optimum performance. Finishes containing 		volatile organic compounds and subject to out-gassing are not acceptable. Locker 		exterior and interior shall be painted the same color.

1.	Powder Coat - Dry Thickness:  1 to 1.2 mils (0.025 to 0.03 mm).

2.	Powder Coat Plus - Dry Thickness:  2 to 2.2 mils (0.05 to 0.055 mm).

3.	Color:  As selected from manufacturer's standard colors.

4.	Special Finish

a. Custom color

b. Anti-Graffiti

c. Anti-Microbial

.

PART 3 - EXECUTION

3.01	EXAMINATION

A.	Do not begin installation until substrates and bases have been properly prepared.

B.	If substrate and bases are the responsibility of another installer, notify Architect of 		unsatisfactory preparation before proceeding.

3.02	INSTALLATION

A.	Install metal lockers and accessories at locations shown in accordance with 			manufacturer's instructions.

B.	Install lockers plumb, level, and square. Work is not to progress until site meets 		necessary conditions.

C.	Anchor lockers to floor and/or wall as recommended by the manufacturer.

D.	Bolt adjoining locker units together to provide rigid installation.

E.	Install sloping tops and metal fillers using concealed fasteners.  Provide flush 			hairline joints against adjacent surfaces.

F.	Install benches by fastening bench tops to pedestals and securely anchoring to the 		floor using appropriate anchors for the floor material.

3.03	ADJUSTING AND CLEANING

A.	Adjust doors and latches to operate without binding.  Verify that latches are 			operating satisfactorily.

B.	Adjust built-in locks to prevent binding of dial or key and ensure smooth operation 		prior to substantial completion.

C.	Touch-up with factory-supplied paint and repair or replace damaged products before 		substantial completion.

3.04	PROTECTION

A.	Protect installed products until completion of project.

END OF SECTION

Penco Products, Inc. reserves the right to vary specifications consistent with a policy of continuous product improvement.