SECTION 10 51 13

METAL LOCKERS

PART 1 – GENERAL

1.01 	SECTION INCLUDES

A. 	Standard Duty Knocked Down Lockers.

B. 	Locker benches.

1.02	REFERENCES

A.	ADAAG - Americans with Disabilities Act, Accessibility Guidelines.

1.03	SUBMITTALS

A.	Submit under provisions of Section 01 33 00.

B.	Manufacturer's data sheets on each product to be used, including:

1.	Preparation instructions and recommendations.

2.	Storage and handling requirements and recommendations.

3.	Installation methods.

C.	Shop Drawings: Show the following:

1.	Dimensioned drawings including plans, elevations, and sections to show locker locations and interfaces with adjacent substrates.

2.	Details of assembly, erection, anchorage and clearance requirements.

D.	Selection Samples: For each finish product specified, supply paint on paper chips in

the chosen color(s), representing manufacturer's color and finish.

1.04	DELIVERY, STORAGE, AND HANDLING

A.	Store products in manufacturer's unopened packaging until ready for installation.

B.	Protect locker finish and adjacent surfaces from damage.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A.	Acceptable Manufacturer: Penco Products, Inc., which is located at: 1820 Stonehenge Drive, Greenville, NC 27858; Tel: 800-562-1000; Fax: 800-248-1555; Email: general@pencoproducts.com; Web: www.pencoproducts.com

B.	Lockers shall be GREENGUARD Gold Certified by UL Environment through the GREENGUARD Certification Program.

2.02 MATERIALS

- Steel: Prime grade mild cold-rolled sheet steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A1008.

- Steel: Sheet steel components shall be fabricated using zinc-coated steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A879.

- Bolts and Nuts: Zinc plated truss fin head bolts and hex nuts.

- Provide only metal lockers fabricated in the United States by a single domestic manufacturer.

2.03	STANDARD DUTY LOCKERS

A.	Standard Duty Lockers:

1.	Acceptable Product: Penco Guardian, knock-down lockers.

2.	Tops, Bottoms, Backs, Sides, and Shelves: 24 gauge sheet steel.

3.	Doors are 16-gauge sheet steel.

4.	Legs: 6 inches (150 mm) high (standard).

5.	No legs (optional).

B. 	Locker Body: Steel specially formed for added strength and rigidity and to ensure 				tight joints at fastening points.

1.	Tops and bottoms with four sides formed 90 degrees, the front offset formed 					to be flush with horizontal frame member.

2.	Shelves with four sides formed to 90 degrees, front edge having a second 					bend.

3.	Hole spacing in locker body construction:  Not exceeding 9 inches (225 mm).

4.	Form door frame members to a channel shape, not less than 16 gauge steel.

5.	Provide vertical door frame members with additional 3/8 inch (9.5 mm) flange 					as a continuous door strike.

6.	Securely weld cross frame members of channel shapes to vertical framing 					members to ensure rigidity, including intermediate cross frame on double and triple tier

lockers.

7.	Optional factory assembly of locker bodies using rivets.

8.	Center partitions:  24 gauge steel vertical partitions, full depth between 						bottom and shelf.

C.	Locker Doors: One piece sheet steel.

- Multi-Point Latch Doors: Full channel formation of adequate depth to fully 		conceal lock bar on lock side, channel formation on hinge side, right angle 		formations across top and bottom, with holes for attaching number plates.

  - Single Point Latch Doors: 14 gauge door reinforced by a full height 3-1/2 inch (89

mm) wide, 18 gauge vertical pan welded to the top, bottom and hinge side flanges and rear of door skin on 12 inch (305 mm), 15 inch (381 mm) and 18-inch (457 mm) wide doors. Provide a horizontal pan for doors wider than 18-inch (457 mm).

- Single Point Latch Doors: 14 gauge door reinforced by a full 18 gauge inner pan welded

to outer door skin on all four sides.

  - Solid outer door, solid inner pan

  - Diamond perforated outer, offset diamond perforated inner pan.

  - 3 inch or 6 inch louvered outer door, mini louvered inner pan.

  - Mini louvered outer, mini louvered inner pan.

- Provide holes for attaching number plates.

- Doors 18 inches (457 mm) or wider by 36 inches (914 mm) or higher:  3 inch (76 mm)

wide 20 gauge full height reinforcing pan welded to inside face of door at 6 inch (150

mm) centers.

- Ventilation:  Flush door front with no exposed louvers and air flow slots located in top and

bottom flanges of door.

D.	Hinges:

1.	Two inch high, 0.074 inch (1.88 mm) thick sheet steel, double spun, full loop,

tight pin, projection welded to door frame and securely fastened to the door with two

aluminum rivets.

a.	Doors over 48 inches (1.066 m) high:  Three 2 inch (51 mm) high five-

knuckle hinges.

b.	All other doors:  Two 2 inch (51 mm) high five-knuckle hinges.

2.	16 gauge, 0.0625 inch (1.58 mm) thick sheet steel, continuous type hinge 				riveted to the frame with steel rivets and welded to the door.

2.04	DOOR HANDLES AND LATCHING

A.	Two Person and Duplex Lockers, 1, 2 and 3 Tier: Multi-point latching:

1.	Classic III Multi-point latching with recessed handles:

a.	Recess finger-lift control handle in door.

b.       Pocket: brushed stainless steel securely fastened to door with two tabs  and a positive tamper-resistant decorative fastener; of depth sufficient to prevent a combination padlock, built-in combination lock, or key lock from protruding beyond door face.

c.	Provide lock hole cover plate for use with padlocks.

d.	Attach 14 gauge formed steel lifting piece to latching channel with one 						concealed retaining lug and one rivet, assuring a positive two-point 						connection.

e.	Handle finger lift:  Molded, sound-deadening, attached with rivet; 							padlock eye for use with 9/32 inch (7.1 mm) diameter padlock shackle.

f.	Latch Clip:  Glass-filled nylon engaging the door frame and holding the 						door shut.

1)	Doors 48 inches (1219 mm), 60 inches and 72 inches (1829 mm) high: Three 	points.

2)	Doors 36 inches (1.22 m) high:  Two points.

g.	Locking Device: Positive, automatic type, whereby locker may be 						locked when open, 	then closed without unlocking.

h.	Firmly secure rubber silencers in frame as required by manufacturer

B.	1, 2 and 3 Tier: Single-point latching.

      - Single-point latching for horizontal bolt or padlock:

        - Recess handle in door.

        - Integral Pocket and Pull:  brushed stainless steel securely fastened to door with two lugs and a positive tamper-resistant decorative fastener.

          - Pocket Depth:  Sufficient to prevent a combination padlock, built-in combination lock, or key lock from protruding beyond door face.

          - Pull:  Formed in pocket.

          - Padlock Staple:  Protruding through pocket.

        - Provide lock hole cover plate for use with padlocks.

        - Locking Device:  11 gauge steel hasp welded to locker frame; include surface for engaging the bolt of a built-in combination or key lock and anti-pry lug and slot to deter prying open when locked.

        - Full depth side stiffeners to be used on single and double tier SPL Defiant II lockers to prevent frame channel and side-torqueing. Used on left ends of a 24-gauge KD body.

        - Firmly secure rubber silencers to locker frame.

2.	Single-Point latching for wrap around/rotary latch locks:

a.	Recess Handle in door

b.	Integral Pocket and Pull:  brushed stainless steel securely 		fastened to door with two lugs and a positive tamper-resistant 			decorative fastener.

1)	Pocket Depth:  Sufficient to prevent a combination padlock, built-in combination lock, or key lock from protruding beyond door 		face.

2)	Pull:	Formed in pocket.

3) 	Pocket punched for built-in lock mounting only. No padlock slot.

c.	Locking Device:  11 gauge steel hasp welded to locker frame; include 				surface for secure wrap around vertical engagement by a built-in rotary-type lock to deter prying open when locked. Hasp must be adaptable to common horizontal throw locker lock and padlock use.

d.	Firmly secure rubber silencers to locker frame.

2.05	INTERIOR EQUIPMENT

- Single Tier lockers 48.5 inch or higher shall have a hat shelf located approximately 9-inches below the top of the locker. If less than 18-inch deep, locker shall have three single prong hooks and one double prong ceiling prong hooks on 9-inch and 12-inch wide lockers. Four single prong hooks are supplied in wider models. Single tier lockers 18 inches or more in depth shall have a coat rod instead of a ceiling hook. 30 inches and 36 inches high lockers shall have three single prong wall hooks and one double prong ceiling hook through 12-inch wide and four wall hooks for 15 inch or wider. 24-inch wide will have two double prong ceiling hooks. 20-inch and 24-inch high openings to have three wall hooks through 12-inch wide and four wall hooks for 15-inch wide or wider. Hooks to be attached with two bolts per hooks.

B.	ADA-Compliant Lockers (Recessed Handles with Multi-Point Latch):

1.	Single Tier Lockers:  Hat shelf 48 inches (1.219 m) maximum off the floor.

2.	Locker Compartment Bottom:  Minimum of 15 inches (230 mm) off the floor, 					or an extra shelf placed 15 inches (381 mm) off the floor for unobstructed forward and

side reach.

3.	Handicapped symbol attached to door.

4.	Hooks and rods as specified for other lockers.

2.06	ACCESSORIES

A.	Number Plates: Provide each locker with a polished aluminum number plate, 2-1/4 				inches (57 mm) wide by 1 inch (25 mm) high, with black numerals not less than 3/8 inch (9.5

mm) high; attach to face of door with two aluminum rivets.

B.	Closed Bases: 18 gauge closed metal front and end bases, finished to match

lockers.

C.	Locks: Built-in flat key locks; control-key to same series.

D. 	Locks: Built-in grooved key locks (pin tumbler); control-key to same series.

E.	Locks: Built-in three-number dialing combination locks capable of at least five 					different combinations changes; provide control key, combination change key, and 				combination control charts.

F. 	Padlocks: Control-keyed, three-number dialing combination type padlocks; provide 				control key. Mechanism must be resistant to “shimming”.

G.	Continuous Sloped Hoods: 18 gauge steel, slope rise equal to 1/3 of the locker 				depth (18.5 degrees), plus a 1 inch (25 mm) vertical rise at front.

1.	Supplied in 72 inch (1829 mm) lengths only.

2.	Slip joints without visible fasteners at splice locations.

3.	Provide necessary end closures.

4.	Finish to match lockers.

H.       Continuous Sloped Hoods: 16 gauge steel, slope rise equal to 1/3 of the locker 				depth (18.5 degrees), plus a 1 inch (25 mm) vertical rise at front.

1.	Supplied in 72 inch (1829 mm) lengths only.

2.	Slip joints without visible fasteners at splice locations.

3.	Provide necessary end closures.

4.	Finish to match lockers.

- Unit Slope Tops for Standard Duty Lockers: 24 gauge steel, slope rise equal to 1/3 of the locker

depth, finish to match lockers.

- Boxed Finished End Panels: Minimum 16 gauge steel formed to match locker depth and

height, 1 inch (25 mm) edge dimension; finish to match lockers; install with concealed fasteners.

- Front Fillers: 20 gauge steel formed in an angle shape, with 20 gauge slip joint 			angles formed in an angle shape with double bend on one leg forming a pocket to provide

adjustable mating with angle filler.

1.	Attachment by means of concealed fasteners.

2.	Finish to match lockers.

- Zee Bases for Knock-Down Lockers without legs: 14 gauge, steel flanged outward at top for

support of lockers, flanged inward at bottom for anchoring to floor.

1.	Height:  4 inches (101 mm).

2.	Height:  6 inches (150 mm).

M.	Recess Trim: 18 gauge steel, 3 inch (75 mm) face dimension.

1.	Vertical and/or horizontal as required.

2.	Standard lengths as long as practical.

3.	Attach to lockers with concealed clips.

4.	Provide necessary finish caps and splices.

5.	Finish to match lockers.

- Benches: Laminated selected hardwood, 1-1/4 inch (31 mm) full finished thickness,

corners rounded and sanded, surfaces finished with two coats of clear lacquer.

1.	Depth: 9-1/2 inches (240 mm) deep.

2.	Depth: 12 inches (305 mm) deep.

3.	Depth: 24 inches (610 mm) deep.

4.	Lengths:  As shown.

- Heavy-Duty Bench Pedestals: Steel tubing with 11 gauge steel flanges welded to

each end, 16-1/4 inches (412 mm) high, finish to match lockers.

- Stainless Steel Free-Standing Bench Pedestals: 2-inch (50 mm) diameter brushed 16 gauge stainless steel formed into a trapezoid, 14 inch (355 mm) wide bottom with two 5/16 inch (7.9 mm) diameter holes, top flange with four 5/16 inch (7.9 mm) diameter holes for fastening to bench

2.07	FABRICATION

A.	Fabricate lockers square, rigid, without warp, with metal faces flat and free of 					distortion.

B.	Knock-Down Lockers: Fabricate lockers on the unit principle, each locker with 					individual door and 	frame, individual top, bottom, back, and shelves, with common 				intermediate divisions separating compartments. Verify dimensions and 						arrangement before fabrication.

C.	Finish: Hybrid Polyester powder coat paint finish electrostatically applied and properly cured to

manufacturer’s specifications for optimum performance. Finishes containing volatile organic compounds and subject to out-gassing are not acceptable.

      - Powder Coat - Dry Thickness:  1 to 1.2 mils (0.025 to 0.03 mm).

      - Powder Coat Plus - Dry Thickness:  2 to 2.2 mils (0.05 to 0.055 mm).

      - Color:  As selected from manufacturer's standard colors.

      - Special Finish

        - Custom color

        - Anti-Graffiti

        - Anti-Microbial

PART 3 – EXECUTION

3.01	EXAMINATION

A.	Do not begin installation until substrates and bases have been properly prepared.

B.	If substrate and bases are the responsibility of another installer, notify Architect of 				unsatisfactory preparation before proceeding.

3.02	INSTALLATION

A.	Install metal lockers and accessories at locations shown in accordance with 					manufacturer's instructions.

B.	Install lockers plumb, level, and square.

C.	Anchor lockers to floor and wall at 48 inches (1.219 m) or less, as recommended by the

manufacturer.

D.	Bolt adjoining locker units together to provide rigid installation.

E.	Install sloping tops and metal fillers using concealed fasteners.  Provide flush 					hairline joints against adjacent surfaces.

F.	Install front bases between legs without overlap or exposed fasteners.  Provide end 				bases on exposed ends.

G.	Install benches by fastening bench tops to pedestals and securely anchoring to the 				floor using appropriate anchors for the floor material.

3.03	ADJUSTING AND CLEANING

A.	Adjust doors and latches to operate without binding.  Verify that latches are 					operating satisfactorily.

B.       Adjust built-in locks to prevent binding of dial or key and ensure smooth operation 				prior to substantial completion.

C.	Touch-up with factory-supplied paint and repair or replace damaged products before 				substantial completion.

3.04	PROTECTION

A.	Protect installed products until completion of project.

END OF SECTION

Penco Products, Inc. reserves the right to vary specifications consistent with a policy of continuous product improvement.