SECTION 10 51 13

METAL LOCKERS

PART 1 – GENERAL

1.01	SECTION INCLUDES

A.	Heavy Duty Welded Fully Framed Duty Lockers.

1.02	REFERENCES

A.	ADAAG – Americans with Disabilities Act, Accessibility Guidelines.

1.03	SUBMITTALS

A.	Submit under provisions of Section 01 33 00.

B.	Shop Drawings: Show the following:

1.	Dimensioned drawings including plans, elevations, and sections to 				show locker locations and interfaces with adjacent substrates.

2.	Details of assembly, erection, anchorage and clearance requirements.

C.	Selection Samples: For each finish product specified, supply paint on paper chips in

the chosen color(s), representing manufacturer's color and finish.

1.04	DELIVERY, STORAGE, AND HANDLING

A.	Store products in manufacturer's unopened packaging until ready for installation.

B.	Protect locker finish and adjacent surfaces from damage.

PART 2 – PRODUCTS

2.01	MANUFACTURERS

A.	Acceptable Manufacturer: Penco Products, Inc., which is located at: 1820 Stonehenge Drive, Greenville, NC 27858; Tel: 800-562-1000; Fax: 800-248-1555; Email: general@pencoproducts.com; Web: www.pencoproducts.com

B.	Lockers shall be GREENGUARD Gold Certified by UL Environment through the GREENGUARD Certification Program.

2.02	MATERIALS

- Steel: Prime grade mild cold-rolled sheet steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A1008.

- Steel: Sheet steel components shall be fabricated using zinc-coated steel free from surface imperfection, capable of taking a high-grade enamel finish and in compliance with ASTM A653.

- Self-Tapping Screws and Rivets.

- Provide only metal lockers that conform to Made in US regulations.

2.03	HEAVY DUTY PERSONAL DUTY LOCKERS

A.	Heavy Duty Lockers: All locker body components made of cold rolled steel specially 		formed for added strength and rigidity and to ensure tight joints at fastening points.

B.	Locker Body Construction: Welded shell.

1.	Bottoms, Tops, and Shelves: 16-gauge steel.

2. 	Sides: 16-gauge solid sides

3.	Backs:  Solid 18-gauge steel. One-piece.

4.  	Doors:

a.	14-gauge steel.

b.	Standard Ventilation:  6-inch (152.4 mm) wide by ¾” high (19.05 mm) 				high horizontal louvers arranged two groups of 6.

c.       Optional Ventilation:  3/4 inch (19 mm) wide by 1-1/2 inch (38 mm) 				high diamond-shaped perforations.

d.	Optional Ventilation: Solid

5.	Tops and bottoms with four sides formed at 90 degrees welded to the fully- framed channel with cast corner pieces.

6.	Shelves with four sides formed to 90 degrees, front and back edge having a second bend.

7.	Locker Body Construction: Welded shell. Tubular frame construction with cast steel interlocks provide a true fully framed locker body construction

8.	Hole spacing in locker body construction: Not exceeding 1.5-inches.

C.	Locker Doors: One-piece sheet steel.

1.	Provide holes for attaching number plates.

D.	Hinges:

1.	Continuous type:  16-gauge piano hinge measuring full height of door. 				Welded to door and attached to locker frame using steel rivets.

E.	Duty Locker Drawer Base with Integral Seat:

1.	78-inch high Duty lockers, provide a 78-inch high locker with a 9-inch or

12-inch front extension to create the drawer base.

2.	90-inch high Duty lockers, provide a 90-inch high locker with a 9-inch or

12-inch front extension to create the drawer base.

3.	16-gauge drawer body, rated at 250 pounds per pair.

a.	Drawer face with integral finger grab and punched to accept built-in spring bolt locker lock, key lock.

4.	Hardwood seat: Laminated selected hardwood, 1-1/4 inch (31.75 mm) full finished thickness, corners rounded and sanded, surfaces finished with two coats of clear lacquer.

1.	Depth: 9-1/2 inches (241.3 mm) or 12-inches (304.8 mm) depending on front extension size

2.	Length: As shown, continuous along adjoining lockers with drawer bases or individual benches for each unit.

2.04	DOOR HANDLES AND LATCHING

A.	Single Tier Lockers:

1.	Ultra Cremone Recessed Handle: Handle shall be a die cast lift handle that

engages the door frame on three sides. The top and bottom frames are engaged with 0.375 inch steel rods, and a 0.25 x 0.36 inch thick center latch engages at the side or left door. Locate handle in the formed stainless steel pocket.

a.       Double door configurations shall consist of a left hinged door secured 				its full length by the right hinged door when latched.

b.       Handle assembly shall be secured to the pocket using a guided

channel. Pocket is secured to the door in two places.

c.	Doors 18” wide or wider shall be reinforced using a 20-gauge vertical pan spot welded at a maximum of 8” on center to the hinge side of the door.

d.	Door shall be reinforced using a 20-gauge horizontal box pan spot-		welded a maximum of 8" on center to the hinge and latch side 			channels, and spot welded a maximum of 8” on center to the door skin 		along the upper and lower pan edge for optimum strength, quiet 			operation and stiffness.

D. ADA Compliant:

1. Ultra Cremone Recessed Handle mechanism shall be ADA complaint requiring

no twisting, pinching or tight grasping and is operable using less than

5lbs of force.

2.05	INTERIOR EQUIPMENT

A.	Heavy Duty Duty Lockers with Doors:

1.	Optional Full Width Shelf - located approximately 13-inches (330 mm) below top of locker.

2.	Optional Coat Rod – using two heavy duty single prong hooks, mounted to shelf and two single prong hooks with a diameter of 0.187 in rear, mounted to shelf.

2.06	ACCESSORIES

A.	Number Plates:  Provide each locker with a polished aluminum number plate, 2-1/4 		inches (57 mm) wide by 1-inch (25 mm) high, with black numerals not less than 3/8 		inch (9.5 mm) high; attach to face of door with two aluminum rivets.

B.       Optional Name Card Holder: Plated steel. Holds 1.25” (31.75 mm) x 2.75” card 		(69.85 mm).

C.	Locks:  Built-in flat key locks; master-key to same series.

D.	Locks:  Built-in grooved key locks (pin tumbler); master-key to same series.

E.	Locks:  Built-in three-number dialing combination locks capable of at least five 			different combination changes; provide master key, combination change key, and 		combination control charts.

F.	Padlocks:  Master-keyed three-number dialing combination type padlocks; provide 		master key.

G.	Continuous sloped hoods:  16-gauge and 18-gauge steel, slope rise equal to 1/3 of the locker depth (18.5 degrees), plus a 1 inch (25 mm) vertical rise at front.

1.	Supplied in 72-inch (1829 mm) lengths only.

2.	Slip joints without visible fasteners at splice locations.

3.	Provide necessary end closures.

4.	Finish to match lockers.

H.	Finished End Panels:  Minimum 16-gauge steel formed to match locker depth and 		height, 1-inch (25 mm) edge dimension; finish to match lockers. Install with 			concealed fasteners.

I.	Vertical Fillers:  20-gauge steel formed in an angle shape, with 20-gauge slip joint 		angles formed in an angle shape with double bend on one leg forming a pocket to 		provide adjustable mating with angle filler.

1.	Attachment by means of concealed fasteners.

2.	Finish to match lockers.

J	Recess Trim:  18-gauge steel, 3-inch (75 mm) face dimension.

1.	Vertical and/or horizontal as required.

2.	Standard lengths as long as practical.

3.	Attach to lockers with concealed clips.

4.	Provide necessary finish caps and splices.

5.	Finish to match lockers.

K.	Benches:  Laminated selected hardwood, 1-1/4 inch (31 mm) full finished thickness, 		corners rounded and sanded, surfaces finished with two coats of clear lacquer.

1.	Depth: 9-1/2 inches (240 mm) deep.

2.	Depth: 12-inches (305 mm) deep.

3.	Depth: 24-inches (610 mm) deep.

4.	Lengths:  As shown.

L.	Heavy-Duty Bench Pedestals:  Steel tubing with 11-gauge steel flanges welded to 		each end, 16-1/4 inches (412 mm) high, finish to match lockers.

M.	Stainless Steel Free-Standing Bench Pedestals:  2-inch (50 mm) diameter brushed 		16-gauge stainless steel formed into a trapezoid, 14 inch (355 mm) wide bottom with 		two 5/16 inch (7.9 mm) diameter holes, top flange with four 5/16 inch (7.9 mm) 		diameter holes for fastening to bench

N.       Interior Equipment

1.	Optional Acrylic mirror: Self-Adhesive, 6-inch (152 mm) x 8-inch (203 mm).

2. 	Optional extra full width shelves. 16-gauge, flanged on four sides and 		attached securely to mounting side rails.

3. 	Optional Center Partition: 16-gauge with flanged front. Attaches securely to shelf and locker bottom or additional lower shelf. (Not for use with footlocker or seat) Mounting holes for half shelves provided on nominal 6 inch centers.

4.	Optional Half Shelf: 16-gauge, used with full height center partition only. Flanged on three sides.

5.	Optional “L” Shaped Half Shelves: 16-gauge, flanged on three sides and attached securely on one side with a side rail, and flanged partition mounted to shelf or half shelf above.

6. 	Optional Security box: 16-gauge lockable door with a 16-gauge side panel. 	The door is attached to the welded frame of the security using a two-point

hinge. The door is locked through a single point latch with a padlock or built in

lock. A door pull shall be provided for use with padlocks. Security box door

frame to be not less than 16-gauge formed to a channel shape. Vertical

members to have an additional flange to provide a continuous door strike.

a.	9-inch wide – for an 18-inch wide locker

b.	12-inch wide – for a 24-inch wide locker

d. 	18-inch wide – for an 18-inch wide locker

c.	24-inch wide – for a 24-inch wide locker

7. 	Optional cell phone/key tray: 8-inch (203 mm) x 2-inch (51 mm) x 2-inch 			(51 mm) mounted to flanged partition side of the half shelf.

8.	Optional sheet steel document sleeve. 16-gauge measuring 12-inches wide 			(horizontal storage) or 9-1/2 inches wide (vertical storage) mounted to flanged

partition side of the half shelf.

9.	Optional Parachute/body armor rack. Formed from ¾ inch diameter, heavy 		wall steel tube. Reinforced by 14-gauge gussets. Mounted to rear of locker

using two mounting rails between corner posts. 180 	pound capacity.

10	Optional Inner Compartment Door, attached to “L” shaped half shelf

11.	Optional Coat Rods using coat rod hooks mounted to shelf.

2.07	FABRICATION

- Fabricate lockers with fully framed construction, 8 male/female tubular insertion points to provide the fully framed locker body, square, rigid, without warp, with metal faces flat and free of distortion.

B.	Fabricate lockers on the unit principle, each locker with individual door and frame, individual top, bottom, back, and shelves. Verify dimensions and arrangement before fabrication.

C.	Finish:  Hybrid Polyester powder coat paint finish electrostatically applied and properly cured to manufacturer’s specifications for optimum performance. Finishes containing volatile organic compounds and subject to out-gassing are not acceptable. Locker 	exterior and interior shall be painted the same color.

1.	Powder Coat - Dry Thickness:  1 to 1.2 mils (0.025 to 0.03 mm).

2.	Powder Coat Plus - Dry Thickness:  2 to 2.2 mils (0.05 to 0.055 mm).

3.	Color:  As selected from manufacturer's standard colors.

4.	Special Finish

a. Custom color

b. Anti-Graffiti

c. Anti-Microbial

PART 3 - EXECUTION

3.01	EXAMINATION

A.	Do not begin installation until substrates and bases have been properly prepared.

B.	If substrate and bases are the responsibility of another installer, notify Architect of 		unsatisfactory preparation before proceeding.

3.02	INSTALLATION

A.	Install metal lockers and accessories at locations shown in accordance with 			manufacturer's instructions.

- Install lockers plumb, level, and square. Work is not to progress until site meets 		necessary conditions.

- Anchor lockers to floor and/or wall as recommended by the manufacturer.

D.	Bolt adjoining locker units together to provide rigid installation.

E.	Install sloping tops and metal fillers using concealed fasteners.  Provide flush 			hairline joints against adjacent surfaces.

F.	Install benches by fastening bench tops to pedestals and securely anchoring to the 		floor using appropriate anchors for the floor material.

3.03	ADJUSTING AND CLEANING

A.	Adjust doors and latches to operate without binding.  Verify that latches are 			operating satisfactorily.

B.	Adjust built-in locks to prevent binding of dial or key and ensure smooth operation 		prior to substantial completion.

C.	Touch-up with factory-supplied paint and repair or replace damaged products before 		substantial completion.

3.04	PROTECTION

A.	Protect installed products until completion of project.

END OF SECTION

Penco Products, Inc. reserves the right to vary specifications consistent with a policy of continuous product improvement.